TRANSMISSION LINES & WAVEGUIDES – ECA1501

EXPERIMENT – 2

Analysis of SWR (Standing Wave Ratio) by Varying Load Impedance and Characteristic Impedance

| Name | K. MONISHA | Reg. No. | 192412296 |
| --- | --- | --- | --- |
| Date | 12.10.2026 | Experiment No. | 2 |

1. AIM

To analyze the standing wave ratio (SWR) of a transmission line by (a) varying load impedance while keeping characteristic impedance fixed, and (b) varying characteristic impedance while keeping load impedance fixed. Verify that SWR = 1 when ZL = Z0, indicating no reflection.

2. SOFTWARE REQUIRED

SCILAB Version 6.1.1

3. THEORY / FORMULAE

Reflection coefficient: Γ = (ZL − Z0) / (ZL + Z0)

Magnitude of reflection coefficient: |Γ| = abs(Γ)

Standing Wave Ratio: SWR = (1 + |Γ|) / (1 − |Γ|)

For positive real impedances, |Γ| is between 0 and 1. When ZL = Z0, Γ = 0 and SWR = 1.

4. PROCEDURE

Open SCILAB software and create a new file.

Enter the characteristic impedance and load impedance values.

Calculate the reflection coefficient and its magnitude.

Calculate SWR using the formula given above.

Run the program and compare the simulation values with the theoretical values.

Repeat for each impedance value in both test cases.

5. TEST CASE 1 – VARYING LOAD IMPEDANCE (ZL)

Fixed characteristic impedance: Z0 = 50 Ω. Vary load impedance ZL = 25, 50, 75 and 100 Ω.

| S. No. | Z0 (Ω) | ZL (Ω) | ¦Γ¦ (theory) | SWR (simulation) | SWR (theory) |
| --- | --- | --- | --- | --- | --- |
| 1 | 50 | 25 | 0.3333 | 2.000 | 2.000 |
| 2 | 50 | 50 | 0.0000 | 1.000 | 1.000 |
| 3 | 50 | 75 | 0.2000 | 1.500 | 1.500 |
| 4 | 50 | 100 | 0.3333 | 2.000 | 2.000 |

SCILAB PROGRAM – TEST CASE 1

clc;
clear;
close;

// Test Case 1: Fixed characteristic impedance, varying load impedance
Zo = 50;
ZL_values = [25 50 75 100];

printf("Test Case 1: Zo = %g ohms\\n", Zo);
printf("ZL (ohms)\\t|Gamma|\\t\\tSWR\\n");

for k = 1:length(ZL_values)
    ZL = ZL_values(k);
    Gamma = (ZL - Zo) / (ZL + Zo);
    ampGamma = abs(Gamma);
    SWR = (1 + ampGamma) / (1 - ampGamma);
    printf("%g\\t\\t%.4f\\t\\t%.3f\\n", ZL, ampGamma, SWR);
end

OUTPUT SCREENSHOT SPACE – TEST CASE 1

6. TEST CASE 2 – VARYING CHARACTERISTIC IMPEDANCE (Z0)

Fixed load impedance: ZL = 75 Ω. Vary characteristic impedance Z0 = 25, 50, 75 and 100 Ω. This follows the values specified in the AIM section.

| S. No. | ZL (Ω) | Z0 (Ω) | ¦Γ¦ (theory) | SWR (simulation) | SWR (theory) |
| --- | --- | --- | --- | --- | --- |
| 1 | 75 | 25 | 0.5000 | 3.000 | 3.000 |
| 2 | 75 | 50 | 0.2000 | 1.500 | 1.500 |
| 3 | 75 | 75 | 0.0000 | 1.000 | 1.000 |
| 4 | 75 | 100 | 0.1429 | 1.333 | 1.333 |

SCILAB PROGRAM – TEST CASE 2

clc;
clear;
close;

// Test Case 2: Fixed load impedance, varying characteristic impedance
ZL = 75;
Zo_values = [25 50 75 100];

printf("Test Case 2: ZL = %g ohms\\n", ZL);
printf("Zo (ohms)\\t|Gamma|\\t\\tSWR\\n");

for k = 1:length(Zo_values)
    Zo = Zo_values(k);
    Gamma = (ZL - Zo) / (ZL + Zo);
    ampGamma = abs(Gamma);
    SWR = (1 + ampGamma) / (1 - ampGamma);
    printf("%g\\t\\t%.4f\\t\\t%.3f\\n", Zo, ampGamma, SWR);
end

OUTPUT SCREENSHOT SPACE – TEST CASE 2

7. RESULTS

Test Case 1: For Z0 = 50 Ω, SWR values for ZL = 25, 50, 75 and 100 Ω are 2.000, 1.000, 1.500 and 2.000 respectively. The minimum SWR of 1 occurs when ZL = Z0 = 50 Ω.

Test Case 2: For ZL = 75 Ω, SWR values for Z0 = 25, 50, 75 and 100 Ω are 3.000, 1.500, 1.000 and 1.333 respectively. The minimum SWR of 1 occurs when Z0 = ZL = 75 Ω.

Note: The source prompt contains conflicting ranges in its descriptions. This report uses the explicit AIM values for each test case: ZL = 25, 50, 75, 100 Ω in Test Case 1, and Z0 = 25, 50, 75, 100 Ω with fixed ZL = 75 Ω in Test Case 2.